Specification

# Combo’Bedkit

Vertical bed head protection

- Description : double vertical bed head protection (Combo’Bedkit as manufactured by SPM) from slightly textured (Optimixt grain as manufactured by SPM) and antibacterial PVC achieving Bs2d0 fire rating with solid colour. It comprises vertical PVC profile to snap-fix onto aluminium rail with 2 end pieces. It projects 75 mm, is 750 mm high and 100 mm wide. These parts fit together on 2 rubber absorbers (25 mm thick)

- Environment: no heavy metals and no substance potentially subjected to any REACH restriction are used in its manufacture, nor PBT/BPA. The calcium-zinc thermal stabilisation process is used. The emission level of volatile substance in inside air has been tested according to ISO 16000-6 and is very low (A+) according to the French regulation (23 March 2011 nr 2011-321 Decree and 19 April 2011 Order). 100% of the product are recyclable.

- Colour: selected by architects from manufacturer’s standard range.

- Installation method: fixes aluminium rail onto the rubber absorbers then the end pieces and snap-fixes onto the PVC profile.