GRADUS Lafite Connect Space is textile, decorative, antistatic floorcoverings available in tiles 50x50cm and planks 25x100cm. It consists in a 540g/m² tufted loop pile layer of solution died and space died nylon on a non-woven polyester backing and  a stabilized bitumen backing. The overall thickness is 5,9mm.

GRADUS Lafite Connect Space has a very good wear resistance thanks to its 170602 points/m² and an impact sound insulation ΔLw=22dB.

GRADUS Lafite Connect Space is a  looselay floorcovering with a Bfl-s1 class for smoke emission according to EN 13501-1.