GRADUS Stratus is a textile, decorative, antistatic floorcoverings available in tiles 50x50cm. It consists in a 615g/m² tufted loop pile layer of 100% solution died AQUAFIL ECONYL SDN on a non-woven polyester backing and  a stabilized bitumen backing. The overall thickness is 6.8mm.

It has a very good wear resistance thanks to its 157750 points/m² and an impact sound insulation ΔLw=23dB.

GRADUS Stratus is a looselay floorcovering with a Bfl-s1 class for smoke emission according to EN 13501-1.