CREATION 30 LOCK

CREATION 30 LOCK is a synthetic, decorative, flexible, antistatic, abrasion group T with beveled edges product. 
CREATION 30 LOCK is a 4mm thickness product, it consists of a transparent wear layer 0,3mm, a design film and a compact under layer. This product is compressed and grained at high pressure.

CREATION 30 LOCK RESISTANCE

CREATION 30 LOCK is treated with polyurethane treatment PUR+ that provides easy to maintain properties and improved resistance to scratches.

CREATION 30 LOCK EASY INSTALLATION

CREATION 30 LOCK is a looselay interlocking system that allows

- installation with no adhesive (interlocking system UNILIN)

- rapid installation

- few tools : cutter and tapping block

- few subfloor preparation

  - CREATION 30 Lock allows unevenness absorption of 1mm max every 20cm

  - the level of flatness must be 5mm max under the 2m rule

CREATION 30 EASY RENOVATION

CREATION 30 Lock is compatible with seamless tile, concrete, PVC, wood floor (if subfloor rigid & stable).