SAFYDOOR

# SAFYDOOR

DOOR FINGER PROTECTION SYSTEM

Description: door finger protection system (Safydoor as manufactured by Gerflor) to prevent fingers being trapped when opening and closing doors, comprising two solid colour, flexible elastomer sections 1.40 m in height, that slot into two pre-drilled aluminium cores. The 7-bellows section is used on the inside door jamb (rabbet side) and can cover a 150-mm gap, with the bellows providing additional extension up to 190 mm. The 3-bellows section is used on the outside door jamb (hinge side) and is designed to move with the door, compressing on opening by 20 to 80 mm.

• Installation method:

Environment: no heavy metals are used in its formulation, including lead or tin or any CMR Cat 1 or 2 substances. This allows off-cuts to be 100% recycled and meets the 7 targets of the HQE building standard.

Colour: selected by architects from manufacturer’s standard range.

Installation sequence: snap-fixes to predrilled aluminium cores screwed to the door and frame.