Tarasafe Plus

TARASAFE PLUS is a  slip resistant safety vinyl floorcoverings, with a PVC wear layer and deeply inlaid mineral crystal particles (patented). It has a thickness of 2.5 mm for an extended durability.

It is reinforced by a glass fibre grid and available in a sheet format.

It is an ideal solution for heavy traffic applications where slip resistance, durability, hygiene and easy maintenance are required

.

TARASAFE PLUS comprises a textile backing for better adherence in wet areas. It helps reducing telegraphing.

It is suitable for most contract & commercial areas.

It is eco-friendly, 100% recyclable, Floorscore® certified and do not contain any heavy metals or CMR 1&2, it is also 100% compliant with REACH.

The product emission rate of volatile organic compounds is < 100 μg/m3 (TVOC after 28 days – ISO 16000 -6).

According to ISO 22196, it has anti bacterial activity against MRSA of 99% after 24h00.