Specification

# Combo’Duo

Wall protection and handrail – 160 mm high

- Description: hand and wall protection rail (Combo’Duo as manufactured by SPM) from 2 mm thick slightly textured (Optimixt grain as manufactured by SPM) and antibacterial PVC achieving Bs2d0 fire rating with solid colour. The 160 mm high system combines a tubular upper part (diameter 38 mm) and a lower wall protection rail slightly textured. The PVC profile snap-fixes onto a continuous aluminium section and finish is provided by slightly textured PVC accessories (wall returns, 90° internal/external angles). The unit is fitted in a PVC spacer. Bactericidal joints are provided for the junctions between profile sections and accessories.

- Environment: no heavy metals and no substance potentially subjected to any REACH restriction are used in its manufacture, nor PBT/BPA. The calcium-zinc thermal stabilisation process is used. The emission level of volatile substance in inside air has been tested according to ISO 16000-6 and is very low (A+) according to the French regulation (23 March 2011 nr 2011-321 Decree and 19 April 2011 Order). 100% of the product are recyclable.

- Colour: selected by architects from manufacturer’s standard range. The colours of the accessories are the same as the profile of the protection handrail.

- Installation method: upper edge of section 0.90 m max. above floor level. Fixing on PVC spacers to be fitted at 0.80 m centres (0.60 m centres in heavy traffic corridors and on light partitions like plasterboard). Wall returns and angles fixing with self-drilling screws.