DECOSMIC

# DECOSMIC

Metallic effect protection and coverings panels

Description: protection and covering panel (Decosmic as manufactured by Gerflor) from rigid and antibacterial PVC achieving Bs2d0 fire rating with solid metallic effect colour and textured surface (with Hydr’X embossment as manufactured by Gerflor). 3 m long, 1.30 m wide, 2 mm thick.

The level required for biocleaning should be validated for very high infectious risk areas such as operating theatres (sector 4) according to the Pasteur Institute of Lille (France).

The chemical and mark resistance should have been proved on the usual cleaning products according to EN26987.

The ease of nuclear decontamination according to ISO 8690 should be excellent.

No heavy metals and no substance potentially subjected to any REACH restriction are used in its manufacture nor PBT or BPA. The calcium-zinc thermal stabilisation process is used. The emission level of volatile substance in inside air has been tested according to ISO 16000 and is under 15μg/m3.100% of the product are recyclable.

Colour: selected by architects from manufacturer’s standard range.

Installation sequence: glue as per manufacturer’s instructions.