ESCORT WITH PVC SHEATH

# ESCORT WITH PVC SHEATH

Diameter 40 mm round handrail

Description: round handrail with antibacterial PVC sheath (Escort as manufactured by Gerflor) with a diameter of 40 mm. The rail consists of an aluminium section covered by a sheath of smooth and antibacterial PVC achieving Bs2d0 fi re rating with solid colour. A protective fi lm is specifi ed to minimise cleaning before acceptance. The section comprises a groove into which brackets, accessories and matching closers slide and lock. Perforated brackets are curved, 40 mm deep and made of satin silver anodised aluminium. Overall projection and depth are  80 / 90 mm. Accessories made of smooth antibacterial PVC (wall returns, external and internal 90° angle pieces or made to measure from 90° to 165°, Rapid'Angle 3-way "universal joint" rotation, slope links) are fitted on the section and fixed with locking screws accessories. Bactericidal joints are provided for the junctions between profile sections and accessories. The profiles can be curved.

No heavy metals and no substance potentially subjected to any REACH restriction are used in its manufacture. The calcium-zinc thermal stabilisation process is used.  The emission level of volatile substance in inside air has been tested according to ISO 16000 and is very low (A+) according to the french regulation (23 march 2011 nr 2011-321 Decree and 19 April 2011 Order). 100% of the product are recyclable.

Colour: selected by architects from manufacturer’s standard range.

Installation sequence: upper edge of section 0.90 m max. above floor level. Fixes to 2-point self-locking perforated aluminium brackets at 1.20 m centres (0.80 m centres in heavy traffic areas and on light partitions like plasterboard).