Specification

# Profil’Inox 50 and 30

90° adhesive corner protectors in brushed stainless steel

- Description: 90˚ adhesive corner protector with 50 or 30 mm wings (internal measurement) (Profil’Inox 50 or 30 as manufactured by SPM), in 10/10th brushed stainless steel approved for food handling areas and provided with two adhesive foam tapes. Wings are lightly beveled. A protective film is specified to minimise cleaning before acceptance.

- Finish: brushed metal

- Installation method: adhesive. SPM universal mastic glue is recommended for stronger fixing.