Specification

# Protecta 2

variable angle adhesive corner protector

- Description : variable angle adhesive corner protector (from 70° to 135°) with flexible strip (Protecta 2 as manufactured by SPM) from smooth and antibacterial PVC, 2 mm thick, achieving Bs2d0 fire rating with solid colour. A protective film is specified to minimise cleaning before acceptance. Wings are 60 mm wide (internal measurement) and the central rounded bead forms a decorative and harmless strip. It is provided with 2 strips of foam adhesive tape for fixing.

- Environment: no heavy metals and no substance potentially subjected to any REACH restriction are used in its manufacture, nor PBT/BPA. The calcium-zinc thermal stabilisation process is used. The emission level of volatile substance in inside air has been tested according to ISO 16000-6 and is very low (A+) according to the French regulation (23 March 2011 nr 2011-321 Decree and 19 April 2011 Order). 100% of the product are recyclable.

- Colour: selected by architects from manufacturer’s standard range.

- Installation method: adhesive. SPM universal mastic glue is recommended for stronger fixing.