Specification

# Safy’corner

90° adhesive cushioning corner protector

- Description: 25 mm wide wings (internal measurement) 90° adhesive cushioning corner protector (Safy’corner as manufactured by SPM) comprising a 10 mm thick smooth and flexible elastomer section with solid colour, covered with a 1 mm thick transparent protective film to reduce cleaning throughout its service life. It is provided with 2 strips of foam adhesive tape for fixing. It has an impact absorption capacity of 38%, thus minimising the risk of injury.

- Environment: no heavy metals are used in its formulation, nor PBT/BPA. 100% of the product are recyclable.

- Colours: selected by architects from manufacturer’s standard range.

- Installation method: adhesive. SPM universal mastic glue is recommended for stronger fixing.